Waves, DC Circuits     PHYS 501    Homework 1

NAME:____________________________      (12 points possible)

1.  For each frequency, calculate the wavelength in meters.  If it is a valid amateur frequency, put which band it's in (17m, etc),  NOTE SIGNIFICANT DIGITS

http://www.arrl.org/files/file/Regulatory/Band%20Chart/Band%20Chart%20-%2011X17%20Color.pdf

Frequency (MHz)                   wavelength(m)                   "band" (if relevant)

a.  49.00      _____________________________________________________

b.  52.525   _____________________________________________________

c.  28.50      _____________________________________________________

d.  221.15    _____________________________________________________

e.  146.52    _____________________________________________________

f.   444.825  _____________________________________________________

g.  450.7125 _____________________________________________________

h.  14.300    _____________________________________________________

2.   An automobile battery generally supplies 12V.  Is that AC or DC?  _________.

If your transceiver transmits 100W, how much current would that require from your car battery?  (but of course your transmitter is not 100% efficient, so the fuse had better be larger than that…)   (power (W) = I x V)  (always show your work)

_____________________________________________________________A

3.   If a gamma ray has a wavelength of 1 nm, what would its frequency be (Hz) (use scientific notation).   c = 3 x 108 m/s

_____________________________________________________________Hz

4.  If a circuit has a total resistance of 2 M  and a current of .5 A, what is the voltage?

________________ V           and the power would be  _____________________W

5.  If you take the circuit of (4) and put two 2 M  resistors in series, what would the new current be?

________________ A          and the total power would be  _____________W

6.  If you take the circuit of (4) and put two 2 M  resistors in parallel, what would the new current be?

_______________ A          and the total power would be  _____________W

What would be the net effective resistance?    ___________M

7.  You are going on a camping trip into the woods.  Which kind of battery would you want in your handy-talkie? _____ Why?______________________________________

___________________________________________________________________

a.  Nickel-cadmium	b.  iron-pyrite

c.  lead-acid	d.  lithium-ion

8.  What is the effective resistance of a circuit with 280V and current of 60A?  (hybrid car)

________________________;  what is the power? _________________________

9.    What is the voltage across a 50  resistor if a current of 2 A flows through it?

________________________V;  what is the power? _________________________

10.  Most homes use an average of 2-3 KW of power.  Over the course of a day, how many kilowatt-hours would be used if the average is 3 KW?  _______________

How many KWH would be used in a 30 day month?  __________________________

(a Kilowatt-hour is a unit of energy, a rate of 1 Kilowatt (1000 J/s) used steadily for one hour.  So 1 KWH = 3.6E6 Joules).

11.  Check your electricity bill if you pay it yourself (if you live in a dorm, ask your folks for a bill).  Choose a summer month if you have one.  Month/year chosen:  _____________

How many KWH did you use that month?  _____________

What was your average rate of usage (KW)? (divide by the number of hours in a month) _______________________________________

What was the price you paid per KWH?_______________

12.   A solar cell that's 1 m square can deliver about 40W.  Assuming its daily summer output is equivalent to 6 hours of max power, how many square meters do you need for your house?  (hint – how many KWH per day does each cell provide?)

______________________________